SPECIFICATION

Micro Dynamic Speaker

器

RoHS Compliance & Halogen Free

Product    :   17*12*2.35 Speaker

GETTOP P/N:   SAS171223PA**

Version     :   P/0

| Designed by | Checked by | Confirmed by |  | Approved by | Released date |
| --- | --- | --- | --- | --- | --- |
| Jianfeng Zhang | Zhenyu Shen | Chao Gao | Ethan |  | 2025.5.13 |

ISO/TS 16949 : 2009

ISO 9001 : 2008

ISO 14001：2004+Cor1 : 2009

BS OHSAS 18001 : 2007

QC080000 :2005

ISO/TS 16949 : 2009

ISO 9001 : 2008

ISO 14001：2004+Cor1 : 2009

BS OHSAS 18001 : 2007

QC080000 :2005

Revision History

| Date | Version | Description | Controller |
| --- | --- | --- | --- |
| 2025-05-13 | P/0 | Initial Release | Jianfeng Zhang |

目 录

1.	Scope	4

2.	Mechanical Layout and Dimensions	4

2.1 Main Dimensions	4

2.2 Appearance drawing	4

3	Electro-Acoustic Characteristics	5

3.1	Typical Electroacoustic Parameters	5

3.2	Frequency Response	6

3.3	THD	7

3.4	HOHD	8

- Scope

- This specification is the basic technical specification for SAS171223PA** products of Gettop Acoustic Co.,Ltd, the device meets ROHS、REACH and other environmental requirements, the control of hazardous substances meets the requirements of Gettop’s latest of “Environmental Management Material Standard”.

- Mechanical Layout and Dimensions

- 2.1 Main Dimensions

-

+

Remark：

1、“*”Critical Dimension

2、Unmarked tolerance:±0.15mm

3、Dimensions with（）are reference dimension

- Unit：mm

- 2.2 Appearance drawing

- Electro-Acoustic Characteristics

  - Typical Electroacoustic Parameters

| No. | Parameter | Technical Specifications | Unit | Conditions |
| --- | --- | --- | --- | --- |
| 1 | DC Resistance | 7±10% | Ω |  |
| 2 | Rated AC Impedance | 8±15% | Ω | @2kHz |
| 3 | Rated Power | 1.7 | W |  |
| 4 | Maximum Short Time Power | 2.5 | W |  |
| 5 | Resonance Frequency | 1010±10% | Hz | in 0.8cc back case (measured @1.7W) ,25℃ |
| 6 | Sound Pressure Level-1 | 97±3 | dB | @2kHz /3.7V/10cm SC Baffle in 0.8cc back case |
| 7 | Sound Pressure Level-2 | 84+/-3 | dB | @500Hz/3.7V/10cm SC Baffle in 0.8cc back case |
| 8 | Listening condition | No Audible Rub & Buzz |  | 3.7Vrms（1.7W）, in 0.8cc back case 100Hz~2kHz；sine wave ，time：1~2s；2cycles |

  - Frequency Response

Test condition：sine wave, 100～20KHz, 1/12 OCT, 3.7V;0.8cc back case，baffle

FR

| FR（Hz） | Upper |  | FR（Hz） | Lower |  |
| --- | --- | --- | --- | --- | --- |
|  | Before Reliability | After Reliability |  | Before Reliability | After Reliability |
| 300 | 78 | 79 | 300 | 72 | 71 |
| 500 | 87 | 88 | 500 | 81 | 80 |
| 1000 | 105 | 106 | 1000 | 99 | 98 |
| 1200 | 105 | 106 | 1200 | 99 | 98 |
| 2000 | 100 | 101 | 2000 | 94 | 93 |
| 6000 | 99 | 100 | 6000 | 91 | 90 |
| 10000 | 99 | 100 | 10000 | 91 | 90 |

FR Limit

  - THD

Test condition：sine wave, 100～20KHz, 1/12 OCT, 3.7V;0.8cc back case，baffle

THD

| THD（Hz） | Before Reliability | After Reliability |
| --- | --- | --- |
| 200 | 35 | 45 |
| 400 | 35 | 45 |
| 1000 | 20 | 26 |
| 2000 | 6 | 7 |
| 10000 | 6 | 7 |

THD Limit

  - HOHD

Test condition：sine wave, 100～20KHz, 1/12 OCT, 3.7V;0.8cc back case，baffle

HOHD

| R&B（Hz） | Before Reliability | After Reliability |
| --- | --- | --- |
| 200 | 1 | 1.5 |
| 1200 | 1 | 1.5 |
| 2000 | 0.5 | 0.75 |

HOHD Limit