FEATURES

Flexible configuration

✓ Support 4-distinct D-ToF illumination

✓ Support high frequency CW-ToF

illumination

✓ Support CMOS and LVDS input interface

✓ Support I2C and SPI interface

Driven capability

✓Up to 200MHz input signal for CW-TOF

✓0.5nS single narrow pulse output for D-TOF

✓ Up to 12V VSCEL supply voltage

✓ Up to 6A output peak current per channel

Packages

✓ OPN7020C

- 36-ball WLCSP (0.4mm pitch)，3.7mm × 1.7mm

GENERAL DESCRIPTION

The OPN7020 is a smart VCSEL laser diode driver

with high performance and high efficiency, which is

optimized for both direct Time of Flight (ToF) and

indirect ToF 3D sensing application.

The OPN7020 fulfills up to 200MHz frequency

continue wave output driven capability with

configurable current limit function for CW-ToF

illumination application. The OPN7020 support 4-

distinct separately control single narrow pulse

generator, typically use in D-ToF illumination

application. A 12-bits ADC is integrated to

implement comprehensive abnormality detection

and protection scheme, specifically for laser eye

safety protection, temperature monitoring and

output current sensing. An advanced feature called

Smart Correction is designed to precisely adjust

output timing against temperature variance.

The OPN7020 supports both single-ended and

differential signaling for input interface. It supports

both I2C and SPI as control interface.

Comprehensive abnormality detection and

protection scheme

- Over current and over temperature detection and protection

- Output short to ground protection

- VCSEL diffuser-error detection (PD current and ITO resistance) for Laser eye safety protection

- Output current sensing

- Internal temperature monitoring

- External temperature monitoring

Applications

- Pulse-ToF illumination

- CW-ToF illumination

- LIDAR illumination