FEATURES

▪ Imaging Array

− 100 × 100 Array

− 1/6 Optical Format

− Pixel Pitch: 16.8 µm

− Frame Rate: up to 120 fps

▪ Interface

− Control Interface: CCI(I2C)

− Data Interface: MIPI CSI-2

▪ 3.3V Single Power upply

▪ Low Power onsumption with Scalability − <50mW @ 30fps

APPLICATIONS

▪ 3D Sensing

▪ 3D Face & Gesture Recognition

▪ 3D Gaming and AR/VR Solutions

▪ Security,Surveillance,Industry

# GENERAL DESCRIPTION

The OPN8001 Time-of-Fight  (ToF) sensor for 3D sensing   is  one   key   product  of  OPNOUS  ToF solutions, which offers  100×100 resolution data at frame rates up to 120 frames per second (fps). The device  combines  ToF  sensing  with  an  optimally- designed  analog-to-digital  converter  (ADC)  and  a versatile,   programmable   timing   generator   (TG). OPN8001 is realized upon NIR wave length (850 nm and 940 nm) with an industry leading accuracy. The power   consumption   of   OPN8001   achieves   the lowest level in the industry, which will benefit many portable and energy saving applications. The device supports MIPI CSI-2 interface.

▪ Leading Accuracy in the Industry

− ≤ 1% of Distance (0.15m ～5m @ 30fps)

▪ On-chip Timing Generator

▪ One System Clock for All Range

Measurement with Wide Range Flexibility

▪ Multi-sensor Capability

▪ Micro Lens Availability with Advanced Optical System Solution Capability

▪ Operating Temperature: -20℃ ~ 60℃

▪ Security,Surveillance,Industry Automation

▪ Drone, AGV and Robot Sensing

▪ Automotive Self-driving Sensing