# General Description

OPNM8501B is a tiny 3D camera module, based on OPNOUS Time-of-Flight  (ToF)  technology  using  VCSEL  illumination. The high integration, low power consumption and high precision make this module ideal for depth sensing applications.

# Key Specifications

| Parameter | Description |
| --- | --- |
| Sensor | OPN8001D, global shutter |
| Resolution | 100 * 100 |
| Pixel size | 15um |
| Sensor size | 1/6’’ |
| Dimensions | 15.00mm * 7.30mm * 3.93mm |
| Frame rate | Up to 60fps |
| Measurement range | Up to 1 0 m |
| FOV | 48°(H) * 48°(V) |
| Distortion | <2.5% |
| Illumination | 940nm, 3W |
| Input clock | 27Mhz |
| Power supply | Sensor: 3.3V single power supply, >=300mA VCSEL: 4.6V, >= 1A |
| Power consumption | 300mW. Typ @ 15fps |
| Depth accuracy | <= 1% / <=1cm |
| Interface | MIPI CSI-2, 2 lanes |

1

# Typical Applications

Face recognition and face motion tracking

3D reconstruction

AR/VR

Hand and finger tracking for gesture control and interaction with virtual objects

Visual support for robot grippers

Localization, tracking and identification of individuals (humans, animals)

People counting and motion analysis

# Module Dimensions

Module connector shown in above Figure:

Part Number of module connector is AXE630124.

Mating connector is AXE530127.

# Pin Definitions

| Pin No. | Name | Description |
| --- | --- | --- |
| 1 | AVDD_3V3 | 3.3V Power Supply |
| 2 | VCSEL_4V6 | VCSEL Supply |
| 3 | AVDD_3V3 | 3.3V Power Supply |
| 4 | VCSEL_4V6 | VCSEL Supply |
| 5 | DV33 | 3.3V Power Supply |
| 6 | VCSEL_4V6 | VCSEL Supply |
| 7 | SLV_LSB | Address Select |
| 8 | GND | Ground |
| 9 | GND | Ground |
| 10 | SPI_MOSI | SPI Data In |
| 11 | MCLK | Reference Clock,27MHz |
| 12 | SPI_MISO | SPI Data Out |
| 13 | GND | Ground |
| 14 | SPI_SCLK | SPI Clock |
| 15 | CSI_D0_P | CSI-2 non-inverting data output of data lane 0 |
| 16 | SPI_nCS | SPI CS |
| 17 | CSI_D0_N | CSI-2 inverting data output of data lane 0 |
| 18 | GND | Ground |
| 19 | GND | Ground |
| 20 | SYNC_IN | Sync input |
| 21 | CSI_CLK_P | CSI-2 non-inverting clock output |
| 22 | ERR | Error indicator |
| 23 | CSI_CLK_N | CSI-2 inverting clock output |
| 24 | I2C_SCL | CCI, Camera control interface |
| 25 | GND | Ground |
| 26 | I2C_SDA | CCI, Camera control interface |
| 27 | CSI_D1_P | CSI-2 non-inverting data output of data lane 1 |
| 28 | GND | Ground |
| 29 | CSI_D1_N | CSI-2 inverting data output of data lane 1 |
| 30 | RESET_N | Reset (active low) |